### PE Jaw Crusher

Link:  Gallery     Get this product price

As the domestic leading manufacturer in crushing and screening area, our products have reached world-level advanced standard. PE Jaw Crusher, as the first choice of primary crushing, can be divided into coarse crushing and fine crushing, the feeding size is 125mm~750mm.

#### Features of PE Jaw Crusher

High crushing ratio, Even final size, Simple structure, Reliable working condition, Easy maintenance, Low operating cost.

#### Application of PE Jaw Crusher

PE Jaw Crusher is mainly used in medium size crushing of rocks and bulk material, whose compressive strength is below 320Mpa. Jaw Crusher is widely used in mine, metallurgy, the building, road and railroad construction, hydraulic construction, chemical industry, fire-proofing material and ceramics.

#### Working Principle of PE Jaw Crusher

The plant is composed of main frame, eccentric shaft, flying wheel, toggle plate, jaw plate, tension rod, fixed jaw, movable jaw, and toggle plate can secure the whole body.

The motor transmits power through belt, and drives the moving jaw to do periodic motion towards the fixed jaw by the eccentric shaft. The angle between toggle plate and moving jaw plate increases when moving jaw moves. So the moving jaw moves towards the fixed jaw. The stuff will be crushed during this process. The angle between toggle plate and moving jaw decreases when moving jaw moves down, the moving jaw leaves fixed jaw by pulling rod and spring, the crushed stuff will be discharged from the outlet.

#### Technical Data of PE Jaw Crusher

| Model | Max. Feeding Size (mm) | Discharge Size (mm) | Capacity (t/h) | Motor Power (kw) | Overall Dimensions (mm) | Weight (t) |
| --- | --- | --- | --- | --- | --- | --- |
| PE400×600 | 350 | 40-100 | 15-60 | 30 | 1700×1732×1653 | 6.5 |
| PE500×750 | 425 | 50-100 | 50-100 | 55 | 2035×1921×2000 | 10.3 |
| PE600×900 | 480 | 65-160 | 60-168 | 75 | 2290×2206×2370 | 16.5 |
| PE750×1060 | 630 | 80-140 | 100-250 | 110 | 2620×2302×3110 | 29 |
| PE900×1200 | 750 | 95-165 | 160-380 | 130 | 3789×2826×3025 | 54.5 |
| PE1000×1200 | 850 | 195-265 | 315-550 | 130 | 3335×3182×3025 | 57 |
| PE1200×1500 | 1020 | 150-300 | 400-800 | 160-220 | 4200×3750×3820 | 100.9 |

Note: Any change of PE Jaw Crusher technical data shall not be advised additionally.

Our Senior Manager

| Kevin | Johnny | Peter |
| --- | --- | --- |

Contact Us

| Shanghai | Zhengzhou |
| --- | --- |
| Tel: +86-21-58385016 Fax: +86-21-58377628 E-mail: sales@kefidinc.com | Tel: + 86-371-67999255 Fax: +86-371-67998980 E-mail: david@kefidinc.com |
| Products | Gallery |
| Mobile Crushing Plant Crawler Mobile Crusher Mobile Jaw Crusher Mobile Impact Crusher Mobile Cone Crusher Mobile VSI Crusher Stationary Crushing Plant Jaw Crusher Impact Crusher Cone Crusher VSI Crusher Aggregate Production Line Stone Crushing Plant Sand Making Plant Grinding Mill Vertical Mill MTW Series Trapezoid Mill European Hammer Mill Micro Powder Mill Trapezium Mill High Pressure Mill Raymond Mill Ball Mill Screening & Washing Vibrating Screen Screw Sand Washing Machine Wheel Sand Washing Machine Feeding & Conveying Vibrating feeder Belt Conveyor | Crushing Mobile Crushing Plant Stationary Crushing Plant Jaw Crusher Impact Crusher Cone Crusher VSI Crusher Aggregate Production Line Grinding Mill Vertical Grinding Mill MTW Series Trapezoid Mill Micro Powder Mill Trapezium Mill High Pressure Mill Raymond Mill Ball Mill European Hammer Mill Screening & Washing Vibrating Screen Sand Washing Machine Feeding & Conveying Vibrating Feeder Belt Conveyor Delivery of Products Technology Workshop Clients Visit and Exhibiton Corporate Culture |