Vibrating Screen

Link:  Gallery     Get this product price

The circular vibrating screen is a new vibrating screen vibrating in a circle, with multiple layers and high efficiency. This screen adopts barrel type eccentric vibration generator, and eccentric block for swing adjustment. Vibrating Screen has a long vibrating line, various screen specifications, reliable structure, and vibrating power and specially designed for quarries stone screening.

#### Features of Vibrating Screen

1. Structure of the circular vibrating screen is new, advanced in technical parameters, high in capacity and screen ratio.

2. This screen adopts eccentric draft and block to adjust swing, operation and maintenance is quite convenient.

3. This screen adopts spring steel knitted screen nest or perforated screen plate, which has long life span and its screen holds are not easy to be stopped.

4. This screen adopts the rubber vibration isolation spring, which has longer life span, low noise and stable resonance region.

#### Application of Vibrating Screen

Vibrating Screen is widely used for products grading for ore, construction materials, traffic, energy, chemistry and other industries, which is ideal equipment for crushing and screening assembly. While small meshed screen is installed, wet and sticky materials can be screened only by adding water to spray.

#### Technical Data of Vibrating Screen

| Model | Screen size (mm) | No. of Layer | Screen mesh (mm) | Max. feed size (mm) | Handling capacity (t/h) | Motor power (kW) | Vibrating frequency (r/min) | Double swing (mm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2YZS1237 | 3700×1200 | 2 | 3-100 | 400 | 10-80 | 11 | 750-950 | 5-9 |
| 3YZS1237 | 3700×1200 | 3 | 3-100 | 400 | 10-80 | 11 | 750-950 | 5-9 |
| 2YZS1548 | 4800×1500 | 2 | 3-100 | 400 | 30-200 | 15 | 750-950 | 5-9 |
| 3YZS1548 | 4800×1500 | 3 | 3-100 | 400 | 30-200 | 15 | 750-950 | 5-9 |
| 2YZS1848 | 4800×1800 | 2 | 3-100 | 400 | 50-250 | 18.5 | 750-950 | 5-9 |
| 3YZS1848 | 4800×1800 | 3 | 3-100 | 400 | 50-250 | 18.5 | 750-950 | 5-9 |
| 4YZS1848 | 4800×1800 | 4 | 3-100 | 400 | 50-250 | 18.5 | 750-950 | 5-9 |
| 2YZS1860 | 6000×1800 | 2 | 3-100 | 400 | 65-586 | 22 | 750-950 | 5-9 |
| 3YZS1860 | 6000×1800 | 3 | 3-100 | 400 | 65-586 | 22 | 750-950 | 5-9 |
| 4YZS1860 | 6000×1800 | 4 | 3-100 | 400 | 65-586 | 30 | 750-950 | 5-9 |
| 2YZS2160 | 6000×2100 | 2 | 3-100 | 400 | 81-720 | 22 | 700-900 | 5-9 |
| 3YZS2160 | 6000×2100 | 3 | 3-100 | 400 | 81-720 | 30 | 700-900 | 5-9 |
| 4YZS2160 | 6000×2100 | 4 | 3-100 | 400 | 81-720 | 30 | 700-900 | 5-9 |
| 2YZS2460 | 6000×2400 | 2 | 3-150 | 400 | 100-810 | 30 | 700-900 | 5-9 |
| 3YZS2460 | 6000×2400 | 3 | 3-150 | 400 | 100-810 | 37 | 700-900 | 5-9 |
| 4YZS2460 | 6000×2400 | 4 | 3-150 | 400 | 100-810 | 37 | 700-900 | 5-9 |

Note: Any change of Vibrating Screen technical data shall not be advised additionally.

Our Senior Manager

| Kevin | Johnny | Peter |
| --- | --- | --- |

Contact Us

| Shanghai | Zhengzhou |
| --- | --- |
| Tel: +86-21-58385016 Fax: +86-21-58377628 E-mail: sales@kefidinc.com | Tel: + 86-371-67999255 Fax: +86-371-67998980 E-mail: david@kefidinc.com |

| Products | Gallery |
| --- | --- |
| Mobile Crushing Plant Crawler Mobile Crusher Mobile Jaw Crusher Mobile Impact Crusher Mobile Cone Crusher Mobile VSI Crusher Stationary Crushing Plant Jaw Crusher Impact Crusher Cone Crusher VSI Crusher Aggregate Production Line Stone Crushing Plant Sand Making Plant Grinding Mill Vertical Mill MTW Series Trapezoid Mill European Hammer Mill Micro Powder Mill Trapezium Mill High Pressure Mill Raymond Mill Ball Mill Screening & Washing Vibrating Screen Screw Sand Washing Machine Wheel Sand Washing Machine Feeding & Conveying Vibrating feeder Belt Conveyor | Crushing Mobile Crushing Plant Stationary Crushing Plant Jaw Crusher Impact Crusher Cone Crusher VSI Crusher Aggregate Production Line Grinding Mill Vertical Grinding Mill MTW Series Trapezoid Mill Micro Powder Mill Trapezium Mill High Pressure Mill Raymond Mill Ball Mill European Hammer Mill Screening & Washing Vibrating Screen Sand Washing Machine Feeding & Conveying Vibrating Feeder Belt Conveyor Delivery of Products Technology Workshop Clients Visit and Exhibiton Corporate Culture |