HC Cone Crusher

Link:  Gallery     Get this product price

HC cone crusher is the international advanced cone crusher that adopts a crushing technology, hydraulic technology, computer control technology and advanced manufacturing technology. HC cone crusher is widely used in mining process and stone's second and tertiary and superfine crushing, that can meet the most demanding conditions and adjust different types and hardness of the ore.

#### Features of HC Cone Crusher

HC cone crusher adopts alloy steel frame body, alloy forged spindle, spiral face gears. Its structural features are small size, light weight, great power, durability. Self storage operating system, fast assembly and disassembly liner, curve chamber design make the cone crusher has the features of large processing capacity, lower operating cost and failure rates.

1. The up and down frame body of crusher is alloy steel, through FEM finite element analysis by computer design, the frame body is more solid.

2. Spindle with alloy forging can withstand the overweight force of crushing. Main shaft, eccentric shaft and eccentric bushing are combined with each other. Through the rotating eccentric bushing can be easily implement multiple eccentricity in order to adjust different ore and mining requirements.

3. Spiral bevel gear surface transmission, big torque and high efficiency. According to the principle of laminated material and characteristics of parietal wear, the curve crushing chamber can ensure the large production, high quality, low cost of wear and tear.

4. According to modular design, lubrication system, hydraulic system can run independently.  Through the chain with the main frame can achieve the protection of oil temperature electric flow, pressure, power, abnormal power supply.

5. Self-operation system can achieve overload protection, over-iron protection, and automatic compensation for liner wear. Through feeding with crowded warehouse, it can make crusher maximize its effectiveness.

#### Working Principle of HC Cone Crusher

Through motor driven small and big bevel gear, crusher can achieve the eccentric rotation of movable cone. When machine is in normal operation, one can through pumping to inject or discharge oil to adjust the discharge opening large or small. When iron through, put hydraulic oil into accumulator. After that, hydraulic accumulator press oil back, then crusher can run normally.

#### Technical Data of HP Cone Crusher

| Model | Main shaft speed(rpm) | Eccentric distance(mm) | Motor power(kw) | Weight(kg) |
| --- | --- | --- | --- | --- |
| HC100S | 360 | 16-25 | 90 | 7100 |
| HC200S | 340 | 16-30 | 150 | 13200 |
| HC300S | 285 | 20-36 | 220 | 19500 |
| HC100 | 395 | 13-28 | 90 | 5400 |
| HC200 | 360 | 16-36 | 150 | 9400 |
| HC300 | 320 | 16-36 | 250 | 14500 |
| HC400 | 290 | 18-50 | 315 | 24200 |

| Model | Chamber Type |  | Max.feeding size(mm) | Capacityt/h:Capacity,Metric tons per hour |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | CSS |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 16 | 19 | 22 | 25 | 29 | 32 | 35 | 38 | 41 | 44 | 48 | 51 | 54 |
| HC100S | EC |  | 240 |  | 80 | 90 | 120 | 160 | 175 | 150 | 130 |  |  |  |  |  |
|  | C |  | 200 | 68 | 75 | 102 | 133 | 117 | 102 |  |  |  |  |  |  |  |
| HC200S | EC |  | 360 |  |  | 122 | 131 | 180 | 243 | 258 | 325 | 343 | 360 | 320 | 269 | 225 |
|  | C |  | 300 |  | 104 | 142 | 152 | 209 | 267 | 284 | 299 | 316 | 277 | 189 |  |  |
|  |  |  |  | 25 | 29 | 32 | 35 | 38 | 41 | 44 | 48 | 51 | 54 | 57 | 60 | 64 |
| HC300S | EC | 450 |  |  | 248 | 264 | 350 | 371 | 468 | 493 | 630 | 550 | 479 |  |  |  |
|  | C | 400 |  | 215 | 295 | 314 | 400 | 423 | 536 | 470 | 418 | 350 |  |  |  |  |
|  |  |  |  | 4 | 5 | 6 | 8 | 10 | 13 | 16 | 19 | 22 | 25 | 29 | 32 | 35 |
| HC100 | EC | 130 |  |  |  |  |  | 48 | 89 | 97 | 104 | 110 | 118 | 127 | 134 | 103 |
|  | C | 90 |  |  |  |  |  | 56 | 93 | 101 | 108 | 116 | 124 | 110 | 74 |  |
|  | M | 64 |  |  |  |  | 46 | 78 | 84 | 80 | 62 |  |  |  |  |  |
|  | MF | 48 |  |  |  | 38 | 70 | 74 | 71 | 56 |  |  |  |  |  |  |
|  | F | 38 |  | 35 | 51 | 52 | 56 | 60 | 50 | 40 |  |  |  |  |  |  |
| EF available |  |  |  | 6 | 8 | 10 | 13 | 16 | 19 | 22 | 25 | 29 | 32 | 35 | 38 | 41 |
| HC200 | EC | 190 |  |  |  |  | 113 | 157 | 169 | 179 | 191 | 207 | 218 | 230 | 218 | 174 |
|  | C | 145 |  |  |  |  | 138 | 149 | 160 | 170 | 182 | 196 | 207 | 197 | 156 | 120 |
|  | MC | 115 |  |  |  | 60 | 147 | 158 | 170 | 182 | 193 | 168 | 152 | 123 |  |  |
|  | M | 90 |  |  |  | 88 | 138 | 149 | 160 | 170 | 161 | 137 | 109 |  |  |  |
|  | MF | 65 |  |  | 64 | 111 | 121 | 130 | 132 | 120 | 97 |  |  |  |  |  |
|  | F | 50 |  | 82 | 87 | 92 | 101 | 108 | 110 | 100 | 81 |  |  |  |  |  |
| EF available |  |  |  | 10 | 13 | 16 | 19 | 22 | 25 | 29 | 32 | 35 | 38 | 41 | 44 | 48 |
| HC300 | EC | 210 |  |  |  | 210 | 290 | 309 | 329 | 355 | 374 | 395 | 415 | 434 | 403 | 320 |
|  | C | 170 |  |  | 106 | 229 | 307 | 327 | 348 | 377 | 397 | 378 | 352 | 276 | 240 |  |
|  | MC | 140 |  |  | 128 | 275 | 296 | 316 | 336 | 363 | 344 | 322 | 254 | 221 |  |  |
|  | M | 105 |  |  | 196 | 292 | 313 | 334 | 356 | 314 | 295 | 233 | 204 |  |  |  |
|  | MF | 80 |  | 120 | 238 | 257 | 276 | 295 | 314 | 277 | 260 | 206 |  |  |  |  |
|  | F | 65 |  | 185 | 201 | 216 | 232 | 248 | 263 | 233 | 218 | 172 |  |  |  |  |
| EF available |  |  |  | 13 | 16 | 19 | 22 | 25 | 29 | 32 | 35 | 38 | 41 | 44 | 48 | 51 |
| HC400 | EC | 300 |  |  | 186 | 355 | 458 | 487 | 526 | 574 | 605 | 635 | 665 | 695 | 663 | 536 |
|  | C | 210 |  |  | 200 | 385 | 504 | 535 | 579 | 611 | 643 | 675 | 643 | 538 | 426 | 371 |
|  | MC | 170 |  |  | 266 | 447 | 478 | 508 | 549 | 580 | 610 | 524 | 488 | 382 | 303 |  |
|  | M | 130 |  |  | 310 | 462 | 494 | 525 | 567 | 527 | 504 | 423 | 389 |  |  |  |
|  | MF | 100 |  | 202 | 387 | 416 | 444 | 472 | 510 | 473 | 408 | 381 |  |  |  |  |
|  | F | 75 |  | 319 | 344 | 370 | 395 | 420 | 454 | 421 | 363 | 339 |  |  |  |  |
|  | EF | 55 |  | 295 | 318 | 313 | 304 |  |  |  |  |  |  |  |  |  |

EF = Extra fine
F = Fine
MF = Medium fine
MC= Medium coarse
M = Medium
C = Coarse
EC = Extra coarse
The capacity is based on granite, the density of aggregate is 1.6t/m3.

Note: Any change of HC Cone Crusher technical data shall not be advised additionally.

Our Senior Manager

| Kevin | Johnny | Peter |
| --- | --- | --- |

Contact Us

| Shanghai | Zhengzhou |
| --- | --- |
| Tel: +86-21-58385016 Fax: +86-21-58377628 E-mail: sales@kefidinc.com | Tel: + 86-371-67999255 Fax: +86-371-67998980 E-mail: david@kefidinc.com |

| Products | Gallery |
| --- | --- |
| Mobile Crushing Plant Crawler Mobile Crusher Mobile Jaw Crusher Mobile Impact Crusher Mobile Cone Crusher Mobile VSI Crusher Stationary Crushing Plant Jaw Crusher Impact Crusher Cone Crusher VSI Crusher Aggregate Production Line Stone Crushing Plant Sand Making Plant Grinding Mill Vertical Mill MTW Series Trapezoid Mill European Hammer Mill Micro Powder Mill Trapezium Mill High Pressure Mill Raymond Mill Ball Mill Screening & Washing Vibrating Screen Screw Sand Washing Machine Wheel Sand Washing Machine Feeding & Conveying Vibrating feeder Belt Conveyor | Crushing Mobile Crushing Plant Stationary Crushing Plant Jaw Crusher Impact Crusher Cone Crusher VSI Crusher Aggregate Production Line Grinding Mill Vertical Grinding Mill MTW Series Trapezoid Mill Micro Powder Mill Trapezium Mill High Pressure Mill Raymond Mill Ball Mill European Hammer Mill Screening & Washing Vibrating Screen Sand Washing Machine Feeding & Conveying Vibrating Feeder Belt Conveyor Delivery of Products Technology Workshop Clients Visit and Exhibiton Corporate Culture |