Vibrating feeder

Link:  Gallery     Get this product price

Vibrating feeder is specially designed for evenly feeding the large size material to the crusher. This series vibrating feeder with Vibrating smooth, reliable operation, low noise, small energy consumption, long life, easy maintenance, light weight, small size, easy adjustment and installation, integrated performance, the closed structure of fuselage can prevent dust pollution.

#### Features of Vibrating feeder

Vibrating feeder with Vibrating smooth, reliable operation, long life;

Exciting force can be adjusted to change at any time and control traffic, adjust stably. Vibratory motor of the Vibrating feeder is excitation source, low noise, small energy consumption, easy maintenance. Simple structure, reliable operation, light weight, small size, the closed structure of fuselage can prevent dust pollution.

#### Working Principle of Vibrating feeder

Vibrating Feeder Composed by the feeding trough, exciter, spring support, transmission gear, etc. the source of the vibration of the feeding trough is the exciter, which composed by two eccentric shafts (driving and passive) and the gear. The motor drive the driving shaft by V-belt, the gear in the driving shaft meshing the passive shaft, driving shaft and passive shaft reverse rotation at the same time, which make the feed trough vibrating , so that make a continuous flow of materials to achieve the purpose of conveying materials.

#### Technical Data of Vibrating feeder

| Model | Max. feeding size(mm) | Handling capacity(t/h) | Power (kW) | Dip angle(º) | Weight(kg) |
| --- | --- | --- | --- | --- | --- |
| GZG70-4 | 200 | 120-170 | 2X0.5 | ~10 | 389 |
| GZG100-4 | 300 | 270-380 | 2X1.1 | ~10 | 762 |
| GZD2400X700 | 400 | 30-80 | 2x1.5 | ~10 | 3000 |
| GZD3000×850 | 450 | 80-120 | 2x3.7 | ~9 | 3900 |
| ZSW3800×960 | 500 | 120-210 | 11 | 0 | 3980 |
| ZSW4200×1100 | 580 | 200-430 | 15 | 0 | 4170 |
| ZSW4900×1100 | 580 | 280-500 | 15 | 0 | 4520 |
| ZSW4900×1300 | 650 | 450-600 | 22 | 0 | 5200 |
| ZSW6000×1300 | 650 | 480-800 | 30 | 0 | 7800 |
| ZSW6000×1500 | 800 | 500-850 | 30 | 0 | 8800 |

Note: Any change of Vibrating feeder technical data shall not be advised additionally.

Our Senior Manager

| Kevin | Johnny | Peter |
| --- | --- | --- |

Contact Us

| Shanghai | Zhengzhou |
| --- | --- |
| Tel: +86-21-58385016 Fax: +86-21-58377628 E-mail: sales@kefidinc.com | Tel: + 86-371-67999255 Fax: +86-371-67998980 E-mail: david@kefidinc.com |

| Products | Gallery |
| --- | --- |
| Mobile Crushing Plant Crawler Mobile Crusher Mobile Jaw Crusher Mobile Impact Crusher Mobile Cone Crusher Mobile VSI Crusher Stationary Crushing Plant Jaw Crusher Impact Crusher Cone Crusher VSI Crusher Aggregate Production Line Stone Crushing Plant Sand Making Plant Grinding Mill Vertical Mill MTW Series Trapezoid Mill European Hammer Mill Micro Powder Mill Trapezium Mill High Pressure Mill Raymond Mill Ball Mill Screening & Washing Vibrating Screen Screw Sand Washing Machine Wheel Sand Washing Machine Feeding & Conveying Vibrating feeder Belt Conveyor | Crushing Mobile Crushing Plant Stationary Crushing Plant Jaw Crusher Impact Crusher Cone Crusher VSI Crusher Aggregate Production Line Grinding Mill Vertical Grinding Mill MTW Series Trapezoid Mill Micro Powder Mill Trapezium Mill High Pressure Mill Raymond Mill Ball Mill European Hammer Mill Screening & Washing Vibrating Screen Sand Washing Machine Feeding & Conveying Vibrating Feeder Belt Conveyor Delivery of Products Technology Workshop Clients Visit and Exhibiton Corporate Culture |